Detailed list and Specification of Machinery lying at Plant

[ A ] PLANT AND MACHINERY

- Twin Screw Extruder for Sheets : 1 Number : Manufacturer : Nuova Covema Spa, Italy

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | Panel / Sheet Line |  |  |  |  |
|  | Width : 1000 mm, Thickness : 4 to 25mm, Output : 125 to 250 Kgs. |  |  |  |  |
|  | Model : Panther 120 / 2 – 22 D, DC Motor Driven |  |  |  | 60.00 KW |
|  | 01 | Twin Screw Extruder |  |  |  |
|  |  | 01 | Capacity | 125 to 250 Kgs. |  |
|  |  | 02 | Screw Diameter | 125 mm |  |
|  |  | 03 | L / D | 1 : 22 |  |
|  |  | 04 | Heating Zone | 5 + 1 |  |
|  |  | 05 | Heating Power |  | 55.00 KW |
|  |  | 06 | Cooling System | By Water |  |
|  | 02 | DC Motor with DC Drive Panel |  |  |  |
|  |  | 01 | Power | 60 KW |  |
|  |  | 02 | Continuously Variable Speed | 200 to 2000 RPM |  |
|  | 03 | Extrusion Head Type |  |  |  |
|  |  | Single Block Head built in special INOX steel suitable to extrude panels of 1000 mm width and 25mm thickness |  |  |  |
|  |  | 01 | Heating and Control Zone | 6 |  |
|  |  | 02 | Total Heating Power |  | 28.00 KW |
|  | 04 | Calibration Unit |  |  |  |
|  |  | Calibrators holding unit with possibility of adjusting length and height complete with all service accessories and three vaccume pumps, Calibrator built in special INOX steel each one of 600 x 1000mm (W) size fully cooled and vaccum sucked with 5HP motor connected for each zone – overall size 1900 x 1000 mm (W) |  |  | 22.00 KW |
|  | 05 | Cooling Tank |  |  |  |
|  |  | Cooling system with forced circulation of chilled water through nebulizers with two outlets for overflow and recycling pump with filter |  |  |  |
|  |  | 01 | Total Power |  | 3.20 KW |
|  |  | 02 | Length of the tank | 2500 mm |  |
|  | 06 | Take-Off Unit |  |  |  |
|  |  | Take-off unit built of a strong electro-welded steel frame N 4 caterpillar belts made of special rubber, ground and rectified 60 schor |  |  |  |
|  |  | Two upper belts with adjustable closing according to the required thickness. Two lower belts adjustable in height |  |  |  |
|  |  | 01 | Useful width | 1000 mm |  |
|  |  | 02 | Contact Length (Maximum) | 2500 mm |  |
|  |  | 03 | Belt movement through a motor reducer in DC for speed regulation |  | 5.50 KW |
|  |  | 04 | Pulling Force (approx.) | 10,000 Kgs. |  |
|  | 07 | Cutting Unit with external dust suction system unit with filtering arrangements |  |  |  |
|  |  | Steel frames sliding on special tracks with adjustable wheels to reach required working height |  |  |  |
|  |  | 01 | Useful cutting width | 1000 mm |  |
|  |  | 02 | Cutting Disk Diameter (Approx.) | 350 mm |  |
|  |  | 03 | Disk Motor Power |  | 3.00 KW |
|  |  | 04 | Motor power (suction system) |  | 5.50 KW |
|  |  | 05 | Suction capacity | 4000 M3 / Hr |  |
|  |  | 06 | Disk Translation motor |  | 2.20 KW |
|  | 08 | Stacking Unit |  |  |  |
|  |  | Complete with oleo dynamic piston to lift the supporting plate |  |  |  |

- Single Screw Extruder for Profiles (without gas vent) : 2 Numbers : Manufacturer : Nuova Covema Spa, Italy

| Srl | Equipment / Machinery |  |  |  |  | Load |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Profile / Section Line |  |  |  |  |  |
|  | Width : 300 mm, Thickness : 40 mm, Output : 60 to 80 Kgs. |  |  |  |  |  |
|  | DC Motor Driven |  |  |  |  | 29.40 KW |
|  | 01 | Single Screw Extruder |  |  |  |  |
|  |  | 01 | Capacity | 60 to 80 Kgs. |  |  |
|  |  | 02 | Screw Diameter | 60 mm |  |  |
|  |  | 03 | L / D | 1 : 25 |  |  |
|  |  | 04 | Heating Zone | 5 + 1 |  |  |
|  |  | 05 | Heating Power |  |  |  |
|  |  | 06 | Cooling System | By Air |  |  |
|  | 02 | DC Motor with DC Drive Panel with coaxial electro fan |  |  |  |  |
|  |  | 01 | Power | 29.40 KW |  |  |
|  |  | 02 | Continuously Variable Speed | 250 to 2500 RPM |  |  |
|  | 03 | Temperature Control Panel |  |  |  |  |
|  |  | Double acting Pyrometers and thermo regulator water cooled through electro valve |  |  |  |  |
|  |  | 01 | Fe – Cost thermocouple |  |  |  |
|  |  | 02 | Heating and Control Zone |  | 6 |  |
|  |  | 03 | Controlled Heating Zone of which are double acting |  | 5 |  |
|  |  | 04 | Presently possibility for extruder motor control and for all units composing line |  |  |  |
|  |  | 02 | Total Heating Power |  |  | 15.00 KW |
|  | 04 | Calibration Unit |  |  |  |  |
|  |  | Calibrators holding unit with possibility of adjusting height complete with all service accessories and one vaccume pumps, Calibrator built in special INOX steel each one of 400 x 200mm (W) size fully cooled and vaccum sucked with 7.5 / 10 HP motor connected for all zones – overall size 1230 x 200 mm (W) |  |  |  |  |
|  | 05 | Cooling Tank |  |  |  |  |
|  |  | Cooling system with forced circulation of chilled water through nebulizers with two outlets for overflow and recycling pump with filter |  |  |  |  |
|  |  | 01 | Total Power |  |  | 3.20 KW |
|  |  | 02 | Length of the tank | 2500 mm |  |  |
|  | 06 | Take-Off Unit |  |  |  |  |
|  |  | Moving on endless belts |  |  |  |  |
|  |  | Upper belt with adjustable closing according to the required thickness. Lower belt adjustable in height |  |  |  |  |
|  |  | 01 | Useful width | 300 mm |  |  |
|  |  | 02 | Contact Length (Maximum) | 1000 mm |  |  |
|  |  | 03 | Total length | 1850 mm |  |  |
|  |  | 04 | DC Motor |  |  | 2.20 KW |
|  |  | 05 | Pneumatic belts movement control |  |  |  |
|  | 07 | Cutting Unit with external dust suction system unit with filtering arrangements |  |  |  |  |
|  |  | Steel frames sliding on special tracks with adjustable wheels to reach required working height |  |  |  |  |
|  |  | 01 | Useful cutting width | 300 mm |  |  |
|  |  | 02 | Cutting Disk Diameter (Approx.) | 250 mm |  |  |
|  |  | 03 | Movement controlled by Archimedean Screw Motor Power | 0.50 HP |  |  |
|  |  | 04 | Motor power (suction system) |  |  | 1.50 KW |
|  |  | 05 | Suction capacity | 30 M3 / Hr |  |  |
|  |  | 06 | Disk Translation motor |  |  | 2.20 KW |
|  | 08 | Stacking Unit |  |  |  |  |
|  |  | Complete with oleo dynamic piston to lift the supporting plate |  |  |  |  |

- High speed Fluidizing Heater Cooler Mixer : 1 Number : Manufacturer : Neoplast Engineering Pvt. Ltd., Ahmedabad

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | Heater Cooler Mixer having starting Kick of 525 Amps. |  |  |  |  |
|  |  | 01 | Container Capacity | 250 Ltrs. |  |
|  |  | 02 | Charging Capacity | 90 Kgs. |  |
|  |  | 03 | Production Capacity | 425 Kg / Hr |  |
|  |  | 04 | Batch Time | 8 Minutes |  |
|  |  | 05 | Drive | 51 / 62 HP | 36/51 KW |
|  |  | 06 | Motor speed in RPM | 1500/3000 RPM |  |
|  |  | 07 | Suitable Cooler | 600 Ltrs. |  |
|  | Cooler Mixer |  |  |  |  |
|  |  | 01 | Container Capacity | 600 Ltrs. |  |
|  |  | 02 | Charging Capacity | 160 Kgs. |  |
|  |  | 03 | Production Capacity | 800 Kg / Hr |  |
|  |  | 04 | Batch Time | 8 Minutes |  |
|  |  | 05 | Drive | 7.5 HP | 7.50 KW |
|  |  | 06 | Motor speed in RPM | 1500 RPM |  |
|  |  | 07 | Total Speed | 75 RPM |  |
|  |  | 08 | Quantity of cooling water / Hr | 3.00 M3 |  |
|  |  | 09 | Cooling Water Temperature | 150C |  |
|  |  | 10 | Cooling Surface Area | 0.50 M2 |  |

- Transfer Printing Machine : 1 Number : Manufacturer : Leonard Kurtz GmbH, Germany

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | UV heated transfer printing machine Model No. : FTF1302 |  |  |  |  |
|  |  | 01 | Roller Diameter | 160 mm |  |
|  |  | 02 | Maximum transfer foil roller diameter | 300 mm |  |
|  |  | 03 | Transfer foil roller core diameter | 25 / 75 mm |  |
|  |  | 04 | Roller width | 1400 mm |  |
|  |  | 05 | Useful width | 1300 mm |  |
|  |  | 07 | Adjustable temperature of top roller | 20 to 1900 C |  |
|  |  | 08 | Height of working level | 910 mm |  |
|  |  | 09 | Minimum panel / sheet length | 350 mm |  |
|  |  | 10 | Maximum panel / sheet thickness | 50 mm |  |
|  |  | 11 | Voltage required | 380 + Neutral |  |
|  |  | 12 | Frequency | 50 Hz |  |
|  |  | 13 | Total Power |  | 9.20 KW |

- Chilling Plant : 1 Number : Manufacturer : Kirlosker Pneumatic Company

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | Chilling Plant : Gas based : Mafron F-22 with cooling tower, Model No. : AC470 |  |  |  |  |
|  |  | 01 | Cylinder | 4 numbers |  |
|  |  | 02 | Capacity | 30 TR |  |
|  |  | 03 | Total power |  | 33.10 KW |

- Chilling Plant : 1 Number : Manufacturer : Freeze King, New Delhi

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | Chilling Plant : Gas based : Mafron F-22 with cooling tower, Model No. : FK1500 |  |  |  |  |
|  |  | 01 | Cylinder | 2 numbers |  |
|  |  | 02 | Capacity | 15 TR |  |
|  |  | 03 | Total power |  | 27.60 KW |

- Diesel Generator Set : 1 Number : Manufacturer : Cater Pillar

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | INTEC MAKE Diesel Generator Set |  |  |  |  |
|  |  | 01 | Horse Power | 272 HP |  |
|  |  | 02 | Capacity | 225 KVA |  |

- Air Compressor : 1 Number : Manufacturer : Ingersoll Rand Make

| Srl | Equipment / Machinery |  |  |  | Load |
| --- | --- | --- | --- | --- | --- |
| 01 | Model No. : 2532C7.5, Srl. No. : 8929012 with V sheeve and fittings |  |  |  |  |
|  |  | 225 Ltrs. Horizontal air receiver Srl. No. 227 with fittings |  |  |  |
|  |  | 01 | Total power | 75 HP | 5.65 KW |

[ B ] TOOLS, TACKLES, and OTHER AUXILARY EQUIPMENTS

- Production Tools (Die) with seizer unit (Calibrator)

### Sheet Line

Set of Die : 2 Number

Width : 990mm, Production size (thickness) : 8mm to 19mm

1015mm Production size (Thickness) : 4mm to 10mm

### Profile Line

Main Door Frame Element, Open Fanlight Element, Single Glazing Element, Picture Frame Element, Double Glazing Element, Drawer Side (4”), Drawer Side (6”), Tongue & Groove Element, Top / Bottom Rail Element, Lock Rail Element, Decorative Architrave Element, Decorative Skirting Element, 1/2 Main Frame Element, Die for export

- Automatic Hopper Loaders (RM Charging system) for Sheet and Profile Line

- Battery Operated Powered Stacker (not in use)

Model : BPST – 1000/10

Capacity : 1,000 Kgs.

Manufacturer : Omega Engineering, Gundlav, Valsad

- Overhead Chain Pulley Block

Capacity : 2,000 Kgs.

Manufacturer : Omega Engineering, Gundlav, Valsad

- Circular Saw

- Grinder

- Ark Welding Machine

- Electrical Installations

  - 11 KV to 440 V step down transformer

  - Panel board for DG Set

  - Panel Board for Sheet line

  - Panel Board for Profile Line

  - Panel Board for High speed mixer

  - Panel Board for Chilling Plant

  - Panel Board for Office lighting

  - Panel Board for Street lighting

- Weighing Machines

Electronic : 2 numbers : capacity 500 and 100 Kgs.

Mechanical : 1 Number 100 Kgs.